Diagram Revision Brief — Post-Defense Panel Comments

Author: Sam   ·   Date: 2026-07-15   ·   For: thesis groupmates

The panel raised two diagram-specific revisions (comments #1 and #3 on the revisions form). Both are re-drawings. LaTeX prose and figure blocks are already in place in methodology.tex pointing at placeholder images; once you produce the finished PNGs with the filenames listed below and drop them into ThesisTemplate/figure/, everything compiles.

Target file format: PNG, ≥ 1600 px wide, 200+ DPI, transparent or white background.

Preferred tool: draw.io / diagrams.net (save the .drawio source next to the PNG in docs/defense/).

Font: Inter or Roboto, size ≥ 12 pt so labels stay legible when scaled to the LaTeX text width.

Diagram A — Channel Transition State Machine (Revision #1)

Panel comment: “Describe the process by which the system determines transitions among LoRa, LTE, GPRS, WiFi, and offline modes.”

Output filename: Fig_5_10_Channel_State_Machine.png (replaces the placeholder example_gray_box.pdf currently referenced by \label{fig:channel_state_machine} in methodology.tex).

Type: finite-state / directed-graph diagram — one node per channel, one edge per transition trigger. Not a fall-through flowchart (that already exists as Figure 5.9, Offline-Buffer Fall-Through Flowchart — do not duplicate it).

States (nodes)

Draw five states in this order, left-to-right, following the priority chain:

1. LoRa (SX1278) — highest priority

2. LTE (A7670E) — cellular preferred

3. GSM 2G (A7670E) — cellular fallback

4. WiFi (ESP32) — depot / hotspot

5. Offline (SPIFFS /tel_buf.log) — buffered

Use a distinct fill colour per state; keep the palette light. Add a small subtitle under each node with the underlying hardware (as in parentheses above) so the reader can map states to the hardware block diagram.

Transitions (directed edges)

Each edge is labelled with the event that fires it. Use the exact source in embedded/hmi_phase1/src/main.cpp as the source of truth. The events are:

| From → To | Trigger event | Source-of-truth |
| --- | --- | --- |
| LoRa → LTE | LoRa ACK timeout (2000 ms) OR LORA_MAX_RETRIES (1) exceeded | LORA_ACK_TIMEOUT_MS, LORA_MAX_RETRIES |
| LTE → GSM 2G | Modem reports RAT_GSM_2G after CGATT / CGACT renegotiation | detectGsmRat(), gsmActiveRat |
| GSM 2G → WiFi | Two consecutive HTTPS POST /telemetry failures on cellular | CH_GSM_NO_SERVER state |
| WiFi → Offline | WiFi.status() != WL_CONNECTED and all cellular attempts failed | bufferTelemetry() call site |
| Offline → any (recovery) | Any live channel returns HTTP 2xx during retry; replay via flushTelemetryBuffer() at MAX_FLUSH_PER_CALL = 5 records/tick | flushTelemetryBuffer() |
| LoRa (return) | loraGatewayRecentlyOk() — gateway ACK within last 90 s clears backoff | lastLoraOkMs, loraGatewayBackoffActive() |
| GSM/LTE (return) | AT+CSQ ≥ 10 and AT+CGACT? PDP context active | gsmReconnect() |

Show recovery edges as dashed arrows to distinguish them from failure edges (solid). Every state must have at least one outgoing edge to the next priority level and a dashed recovery edge back from the offline state (illustrating that any channel can resume from buffered mode).

Annotations

Add three small callout boxes anchored to the diagram (not floating text):

Callout 1 (next to LoRa): “Retry budget: 1 attempt, 2 s ACK window.”

Callout 2 (next to Offline): “Buffer file: /tel_buf.log, ~300 B/record, ~1 MB SPIFFS partition ⇒ ≈ 3 500 records ≈ 4.9 h at 5 s cadence. 32 KB low-space cutoff drops new records to protect the FS.”

Callout 3 (spanning diagram bottom): “Every attempt reuses the same seq and event_id — replayed records are tagged channel_used='offline_replay' at the backend so deduplication is safe.”

Deliverables

Fig_5_10_Channel_State_Machine.png in ThesisTemplate/figure/

Fig_5_10_Channel_State_Machine.drawio in docs/defense/

A light-background variant Fig_5_10_Channel_State_Machine_light.png if the base file is dark-themed (match the pattern of Fig_5_11_..._light.png).

Diagram B — ML Pipeline Redraw (Revision #3)

Panel comment: “Revise the diagram (Figure 5.1) with corrected arrows, greater detail, and a general representation in place of Python.”

⚠  Ambiguity to resolve first: the panel wrote “Figure 5.1” but the current LaTeX numbering has:

Figure 5.1 = concep_diagram.png (Conceptual Framework — no Python content)

Figure 5.11 = Fig_5_11_Anomaly_Pipeline_light.png (ML pipeline — has Python/library icons)

Sub-figure inside 5.11 group = thesis_defense_pipeline_v2.png (also ML pipeline)

The “in place of Python” cue points at Figure 5.11 (or its companion thesis_defense_pipeline_v2.png). Confirm with the panel chair before starting; assume 5.11 by default.

Output filename: Fig_5_11_Anomaly_Pipeline_v3.png (keeps the naming convention; just bump the version suffix). Add a _light.png variant too.

What to fix

1.  “Corrected arrows.” The current diagram has ambiguous edge directions between the primary inference plane and the audit/override inference plane. Redraw so every arrow has a single direction and a labelled event:

IF pass → contextual gate  (label: if_score ≥ τ + δ)

MP override → contextual gate  (label: mp_score ≥ 4.0 ∧ Δfuel < 0  OR  mp_score ≥ 2.0 ∧ Δfuel ≤ −2.0)

Contextual gate → persistence  (label: pass)

Contextual gate → audit-only sink  (label: suppressed)

Persistence → alerts table  (label: combined_score > 0.65 ∧ dedup ok)

2.  “Greater detail.” Add the four suppressors as labelled boxes inside the contextual gate (currently collapsed into one box):

parkedRefuel — rejects fill-up (fuel Δ > +10% while parked)

postRefuel — 5-min settle window after any +5% fuel jump

loadSpikeSuppress — engine load ≥ 55% AND load rise ≥ 25% with small negative fuel Δ

coldStartSuppress — sender-priming interval after ignition

3.  “General representation in place of Python.” Remove any Python-specific icons/labels currently in the PNG and replace with generic algorithm names:

| Remove (Python-specific) | Replace with (general) |
| --- | --- |
| sklearn.IsolationForest | Isolation Forest (one-class ensemble) |
| stumpy.matrix_profile | Matrix Profile (subsequence discord) |
| pandas.DataFrame / numpy.ndarray icons | Feature-vector row (33 dim) |
| pickle model file icon | Serialised model artefact |
| any Python code snippet, sklearn / stumpy / numpy logos, snake icons | plain algorithm names + arrows |

Also drop any file-extension hints like .pkl, .py, .csv in favour of the conceptual data name (e.g. “trained model”, “feature row”).

Deliverables

Fig_5_11_Anomaly_Pipeline_v3.png and _light.png in ThesisTemplate/figure/

Fig_5_11_Anomaly_Pipeline_v3.drawio in docs/defense/

Confirm the LaTeX \includegraphics{...} in methodology.tex around line 1071 points at the new filename (Sam will handle the swap once the PNG lands).

Once your PNGs are dropped in ThesisTemplate/figure/

Ping Sam. He will:

Swap the placeholder example_gray_box.pdf reference to your PNG in methodology.tex (both figures).

Re-flow the caption / paragraph to match the final visual (the placeholder captions and surrounding prose are already written but may need one-line edits once we see the actual layout).

Rebuild the PDF and verify figure numbering renumbered correctly (Fig 5.10 is inserted between the existing 5.9 and 5.11, so cross-references downstream might shift by one).

Reach out on Messenger if any transition trigger in the state-machine table is unclear — the firmware source is the authority, but the phrasing above should be enough to draw the diagram without reading C++.